FMC masterFIP

Production Test Suite
User Manual

Creotech Instruments SA | Apr 2017 | Version 3.0

## Revision Table

| Revision | Date | Author | Comments |
| --- | --- | --- | --- |
| 3.0 | 08/03/2017 | Marek Gumiński | Update to board version V3: removed ADC tests, modified EXT_SYNC tests. |
| 2.0 | 16/11/2016 | Evangelia Gousiou | Changes for V2 version of the board (removed EXT_SYNC_TST) |
| 1.0 | 11/07/2016 | Marek Gumiński | Implemented CERN suggestions |
| 0.1 | 01/06/2016 | Marek Gumiński | Initial version. |

## Table of content

1.	Introduction	- 3 -

2.	FMC masterFIP Board Functionality	- 9 -

3.	PTS Functionality Tests	- 10 -

4.	Log files retrieval	- 11 -

5.	Custom cable preparation	- 12 -

6.	First Time Setup	- 13 -

7.	Testing Procedure	- 15 -

8.	Common Causes of Test Failure	- 17 -

# Introduction

Welcome to the Production Test Suite for the FMC masterFIP board – FMC masterFIP PTS.

The FMC masterFIP Production Test Suite (PTS) is an extension of the original PTS which allows to perform functionality tests on FMC masterFIP boards after manufacturing. The original PTS was intended for testing boards designed for the Open Hardware Repository, but it proved to be adaptable to other boards. It assures that the boards comply with a minimum set of quality rules, in terms of soldering, mounting and fabrication process of the Printed Circuit Boards (PCB).

It is important to note that the FMC masterFIP PTS covers only to the functionality tests of the boards and does not cover any verification or validation tests of the design. This document describes the FMC masterFIP PTS components and its use.

Figure 1: FMC masterFIP PTS view

The main elements of the PTS are listed in Table 1.

Table 1: FMC masterFIP PTS elements

| Item | Comments |
| --- | --- |
| Computer | A computer based on motherboard with 4 line PCIe slots is required |
| Monitor |  |
| Keyboard, Mouse |  |
| Barcode reader | With USB connection to the computer |
| ESD wrist strap | With banana type connection to the computer chassis |
| PCIe extender cable | Two spacers and four screws to fix the PCIe extender board to the computer case |
| SPEC board | Four spacers and height screws are used to fix the board to the computer case |
| FMC masterFIP | Board under test |
| 1x LEMO cable | Any length, min 2 ns recommended |
| 1x LEMO – Dsub9 custom cable | Schematic shown in Figure 9, picture of real component in Figure 6 |
| 1x LEMO – miniDsub9 custom cable | Schematic shown in Figure 9, picture of real component in Figure 6 |
| nanoFIPDiag | nanoFIPdiag version must match FMC masterFIP under test speed version. http://www.ohwr.org/projects/nanofipdiag/wiki |
| USB relay box 1 | FMC Fine Delay version. USB cable provided http://www.ohwr.org/projects/usb-relay-box1/wiki |
| Documentation | This user guide plus the one-page testing procedure |

In terms of software, the computer is equipped with the following:

Table 2: PTS software requirements

Ubuntu Linux 14.04.00 LTS

Python 2.7

The user login is:

Table 3: Computer login

| Username | user |
| --- | --- |
| Password | baraka |

Note that after the software installation the computer should be disconnected from the network and no updates should be allowed.

The following paragraphs provide an overview of the required items.

The FMC masterFIP mezzanine board is tested while mounted on a SPEC carrier board, as Figure 2 shows. The SPEC carrier board provides access to the PCIe interface of the computer. The computer hosts the FMC masterFIP PTS software which provides the automated testing environment.

Figure 2: SPEC-DAC-DDS combination

To facilitate the testing setup, the SPEC carrier is fixed on the computer’s box and a PCIe extender cable is used.

Figure 3: SPEC connected to the PCIe extender

The Relay Box (fine delay version) is controlled by PTS software in order to automate cable reconnections.

Figure 4: USB relay box 1

The nanoFIPdiag is used to verify the WorldFIP bus communication by answering to WorldFIP question frames. Note that the nanoFIPdiag speed version should match the version of the masterFIP under test.

Figure 5 nanoFIPdiag

Custom cables are used to connect the FMC masterFIP to the nanoFIPdiag via the USB relay box.

Figure 6 Custom cables

One standard Lemo00-Lemo00 cable is also required. Any length may be used.

The test setup is shown on Figure 7.

Figure 7: Test setup connections. LEMO-LEMO cable is marked red. Custom LEMO-miniDsub9 is marked blue. Custom LEMO-Dsub9 is marked green.

The complete duration of the test is around five minutes.

In brief, the operator needs to:

- mount the FMC masterFIP board onto the SPEC carrier

- connect the cables between the FMC masterFIP, the USB relay box and the nanoFIPdiag

- run the software

- at certain points of the tests an intervention needs to be done by the operator (e.g.: scan the board’s barcode, check the font panel LED); the interventions are explicitly signaled by the FMC masterFIP PTS software and this manual.

At the end of the functionality tests the operator receives a PASS/FAIL notification. In case of a FAILED board, information is provided on the failing components.

All test results are automatically saved in a folder on the computer.

A board is considered to have passed the PTS testing if it has successfully completed all the functionality tests.

For a FAILED board, you can repeat the test only one more time! If a board FAILs twice, please report to the CERN responsible.

# FMC masterFIP Board Functionality

FMC masterFIP is an interface card for the WorldFIP network in an LPC FMC form-factor.

WorldFIP is a deterministic rad-tol fieldbus used at CERN's LHC for a variety of control systems. Cryogenics, Power Converters, Beam Instrumentation and other critical systems are using WorldFIP for the exchange of data between their sensors and actuators and the control and supervision level. With Alstom phasing out WorldFIP support in 2009, it was decided to insource this technology at CERN.

Figure 8: Bottom, top and front views of the masterFIP board

The FMC masterFIP mezzanine board is tested while mounted on a SPEC carrier board. The SPEC provides FPGA logic, power supplies, memories, clocking resources and interface to the PCIe bus. The mezzanine houses all the WorldFIP specific parts (FielDrive, FieldTR).

There are four different versions of masterFIP boards that correspond to the four different WorldFIP speeds: 31.25 Kbps, 1 Mbps, 2.5 Mbps and 5 Mbps. One can visually distinguish the version by checking the on the resistor indicators on the bottom side of the board as Figure 8 shows.

# PTS Functionality Tests

The PTS consists of a set of six independent tests, each one checking a different part of the FMC masterFIP board. Table 4 gives a short description of each one of them. Note that the same firmware is used for all the tests, loaded at test00.

Table 4: List of tests

| Test | Short Description | Operator's Intervention |
| --- | --- | --- |
| 00 | Loads firmware, tests mezzanine presence, verify speed version | Yes |
| 01 | Test front panel LED’s | Yes |
| 02 | Test WordFIP transmission and FieldDrive control/status lines | No |
| 03 | External Sync input/output tests. | No |
| 04 | 1-Wire Digital thermometer (DS18B20U) communication test. | No |
| 05 | EEPROM (24AA64T) communication test. Writing IPMI data. | No |

# Log files retrieval

Log files with detailed descriptions of the tests are automatically generated and archived in a .zip file called:	<timestamp>_zip_run_<run id>_ FmcMasterFip_<serial number>.zip.

The log files need to be delivered to CERN after completing the tests for all of the boards. To do so, please follow the instructions below:

- Plug the USB memory key in to the computer.

- Wait until Ubuntu mounts automatically the device.

- Using the file explorer navigate to ~/Desktop/pts_log/fmcmasterfip directory.

- Select all the .zip files, right click and select copy.

- Using the file explorer, click on the USB device that appeared on the left column, right click and selecting paste.

- Click on the eject button on the left of the file explorer window and remove the USB key.

- Transfer the data to another computer with Internet access.

Finally, send the .zip file by email to the responsible of tests at CERN.

# Custom cable preparation

Before first setup dedicated cables must be prepared. Cable schematics are shown below. Cable length is not important, as long as it is sufficient to connect to the test setup.

Note that for the PTS, as lengths are very small, there is no need to use the WorldFIP-protocol-specified cable.

The following schematics are made with following markings:

- yellow box – connector (type is given inside the box)

- green box – pins of Dsub connector

- red box – 150 Ohm through-hole resistor

- black wire – coaxial cable (about 30cm)

- blue wire – shield of coaxial cable

Resistors (used for termination) should be soldered close to the Dsub connectors.

Figure 9 Schematic design of custom cables used by FMC MasterFIP PTS .

# First Time Setup

The following list explains how to setup the FMC masterFIP environment for the first time.

- Make sure that the computer is switched off.

- Remove the screws and open the computer cover. Confirm that the USB relay box is mounted on the upper side of the computer cover.

Figure 10: Top side of the computer cover

- Screw the SPEC board on the top side of the computer cover, as Figure 11 shows. Connect one side of the PCIe extender cable to the SPEC. Pass the other side of the cable through the dedicated cut of the computer cover.

Figure 11: SPEC mounted on the computer cover

- Plug the other side of the PCIe extender cable into a PCIe x4 (or higher) computer slot, as indicated in Figure 12. Mount the computer cover back to close the computer box. Make sure the screws are back on their place.

Figure 12: PCIe slot for the extender cable

- Mount the FMC masterFIP board on the FMC connector of the SPEC board.

- Plug the USB cable (connector A) into an available USB slot of the provided computer.

- Plug the barcode-reader into an available USB slot of the computer.

- Connect the monitor, keyboard and mouse to the computer.

- Start the PC.

- Install the FMC masterFIP test environment through the following commands. Please note that the test environment may be installed in any path. The installation sequence then makes sure that the top directory is called fmcmasterfip.

# test environment may be installed in any path

cd ~

# clone pts repository

git clone git://ohwr.org/cern-fip/masterfip/masterfip-tst.git fmcmasterfip

# enter in the directory

cd fmcmasterfip

# run install script

sudo ./install.sh

The install.sh script:

  - makes sure that top directory is called fmcmasterfip

  - installs all required linux packages

  - clones git submodules

  - builds software and kernel modules

  - verifies if modules may be correctly loaded

  - creates test shortcut on desktop

Installation successful!

Creating desktop shortcut

Please make sure that the installation script ended with the following message:

# Testing Procedure

| Place the ESD strap on your wrist. |  |  |
| --- | --- | --- |
| Put the barcode sticker on the Bottom side of the FMC masterFIP under test, in the position indicated with the white square. |  |  |
| Mount the FMC masterFIP under-test on the SPEC board. |  |  |
| Connect the FMC masterFIP with the USB relay box and the nanoFIPdiag using the provided cables, as in Figure 7. Note that the nanoFIPdiag speed version should match the masterFIP speed version. - Connecting Dsub9-LEMO cable: - connect the Dsub9 plug to the nanoFIPdDiag - connect the Positive LEMO plug to CH5 of the USB Relay Box - connect the Negative LEMO plug to CH3 of the USB Relay Box - Connecting miniDsub9-LEMO cable: - connect the miniDsub9 plug to the FMC masterFIP - connect the Positive LEMO plug to AWG IN of the USB Relay Box - connect the Negative LEMO plug to SYNC of the USB Relay Box - Connecting LEMO-LEMO cable: - connect CH1 of the USB Relay Box to the Trigger port of the FMC masterFIP |  |  |
| Switch ON the computer. - Verify that the “Pwr” LED on the SPEC board is ON. This will confirm that the SPEC board is properly plugged. - If the LED is OFF there is a problem with the power supply lines. |  |  |
| After the computer has finished with the booting procedure: - Start the test by double clicking “PTSmasterFIP” icon on the desktop. - New terminal window will appear. - When asked, type the password: “baraka” and [ENTER] |  |  |
| - The program asks for the barcode of the board. - Check that the cursor is on the terminal, press the button/trigger on the barcode reader. - The code will appear on the terminal. Press [ENTER]. - The program will ask for a second barcode, in case the manufacturer has a different serial number system. - Scan the second barcode and press [ENTER], or if there is none, just press [ENTER]. |  |  |
| The program will automatically execute tests 00 to 05. |  |  |
| Tests 00 and 01 ask for the user‘s intervention. - Test 00 will ask the operator to verify the FMC masterFIP speed version by visually checking which of R47, R48, R52, R53 resistors is mount. - Test 01 will ask the operator to verify if LED’s of FMC masterFIP front panel are blinking one-by-one. |  |  |
| At the end the operator is informed of the results of all the tests and is asked if he wants to repeat the whole process. - If no error has occurred, type [n] and then [ENTER]. - In case of error, you could repeat the tests once by typing [y] and [ENTER]. |  |  |
| When prompted to switch off computer: - Type [y] and confirm with [ENTER]. |  |  |

# Common Causes of Test Failure

Once the testing has finished, all the errors will be listed on the screen. Usually, the error message is self-explanatory. If you need detailed information, the test log files can be found in /home/user/Desktop/pts_log/fmcmasterfip.

## Common problems with software setup

Please refer to the following paragraphs regarding issues that might affect each test.

USB calibration box not detected:

- Verify that the USB cable is well connected between the PC and the box itself.

- Verify with lsmod that the cp210x driver is mounted in the kernel.

- Make sure you are using the Ubuntu version 14.04.00 LTS.

SPEC board with FMC masterFIP was not detected

- Verify that the SPEC board is well connected to the PCIe port and the Pwr LED is ON.

- Verify with lsmod that the SPEC and FMC drivers were properly mounted in the kernel.

Figure 13: modules that should be present after the ./install.sh

## Functionality test

### Test 00

- Firmware loading

- FMC presence

- Verification of speed version

  - User is asked to visually verify mounting of resistors R47, R48, R52 and R53 that are used to indicate the speed version

  - Mounting is compared with speed identification detected by FPGA on the SPEC board

  - User is asked to verify if detected speed version matches the expected one

Possible errors:

- More than one SPEC was found

  - Make sure that only one SPEC device is mounted in the test PC.

- No SPEC device was found

- SPEC enumeration failed

  - Make sure that SPEC device was mounted properly.

  - Make sure that all software was successfully build.

  - Make sure that all drivers were loaded successfully.

- Firmware loader failure

- No access to masterfip core

- Wrong bitstream type loaded

  - Make sure that installation procedure was done according to specification

- FMC slot NOT populated

  - Make sure that FMC board is mounted correctly

  - Check FMC soldering

- Resistors mount

  - According to the operator’s visual verification some resistors are not mount but are detected by SPEC FPGA

  - Might be caused by human error or shorting of traces leading to speed indicating resistors

### Test 01

Test01 verifies soldering of the FMC masterFIP front panel LED’s. The user is asked to visually verify if all LED’s are blinking one after the other.

Possible errors:

- Some LED’s may be constantly on or off.

  - Make sure that LED’s are mounted correctly.

  - Make sure the FMC connector is well soldered.

### Test 02

This test performs SSSB231 FieldDrive (IC14) tests. The following points are tested one-by-one as described below.

- FIP communication

  - The masterFIP sends a request frame asking for the presence of the nanoFIPdiag (ID_DAT(147F)).

  - The PTS waits for the response RP_DAT and verifies that all the bytes of the frame received by the masterFIP match the expected ones.

- CD_N line

  - The CD_N line is expected to be inactive during bus idle. The test samples 10 times the fd_rxcdn_i FPGA pin.

  - The CD_N line is expected to be active during frame reception. For this test the masterFIP sends an ID_DAT(147F) which triggers the nanoFIPdiag to reply with an RP_DAT; the fd_rxcdn_i line is expected to be active during the RP_DAT frame reception. In the FPGA a counter is counting the transitions (falling edges) of the fd_rxcdn_i line. The test expects to find the counter increased by two by the end of the RP_DAT frame.

- TX_ERR line

  - The TX_ERR line is expected to be inactive when there is no transmission. The test samples 10 times the fd_txer_i FPGA pin.

  - The TX_ERR line is expected to be inactive during normal frame transmission. For this test the masterFIP sends an ID_DAT(147F) which triggers the nanoFIPdiag to reply with an RP_DAT. The fd_txer_i line is expected to be inactive during this transmission and reception. In the FPGA a counter is counting the transitions (rising edges) of the fd_txer_i line. The test expects to find the counter at zero.

  - The TX_ERR line is expected to be active when the TXD line is stuck low or high. For this test we set the fd_txd_o line to output a constant zero. At the same time we monitor the counter that counts the fd_txer_i transitions and expect it increased.

- WDG_N line

  - The WDG_N line is expected to be inactive when there is no transmission. The test samples 10 times the fd_wdgn_i FPGA pin.

  - The WDG_N line is expected to be inactive during normal frame transmission. For this test the masterFIP sends an ID_DAT(147F) which triggers the nanoFIPdiag to reply with an RP_DAT. The fd_wdgn_i line is expected to be inactive during this transmission and reception. In the FPGA a counter is counting the transitions (falling edges) of the fd_wdgn_i line. The test expects to find the counter at zero.

  - The WDG_N line is expected to be active upon the transmission/reception of a very long frame. For this test we trigger the transmission of a 1024 bytes frame. At the same time we monitor the counter counting fd_wdgn_i transitions and expect it increased.

- RST_N line

  - The RST_N line does not stall the transmission/ reception of data. It resets the TX_ERR and WDG_N lines. Following from the WDG_N test described above, we activate the fd_rst_n_o line and check that the td_wdgn_i goes inactive.

Possible errors:

- Frame format, Frame size, Number of bytes, CRC, Preamble, Data field, Data control field

  - Incorrectly connected cables

  - Wrong speed configuration of SSSB231 (IC14)

  - Wrong speed of the nanoFIPdiag box

  - Faulty transformer (TR1)

  - Faulty SSSB231 (IC14)

  - Faulty soldering of FMC connector

- CD_N during transmission, CD_N during idle, TX_ERR idle, TX_ERR during normal transmission, TX_ERR corrupted transmission, WDG_N during correct transmission, WDG_N during long transmission, WDG_N during idle, RST_N

  - Faulty SSSB231 (IC14)

  - Faulty soldering of IC14

  - Faulty soldering of FMC connector

### Test 03

Lemo connector and buffer SN74LVC8T245PW (IC1) input/output test.

The table below describes the series of tests performed regarding the SN74LVC8T245PW (IC1) and its associated pins. In bold we highlight the signal that mostly defines the output.

During all tests FPGA IO connected to EXT_SYN line is kept in High-Z state with internal pulldown resistor. EXT_SYNC line is connected to USB Relay Box with Lemo cable to channel 1, that is connected to Vref voltage. Voltage value is controlled via USB.

Table 5: Sequence of actions to test the SN74LVC8T245PW (IC1)

| Test# | OEn | EXT_SYNC_DIR | EXT_SYNC_TERM_EN | USB Relay Box | Expected Result | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 0 | 0 (input) | 0 | Low | Read "0" from the FPGA EXT_SYN pin forced by USB Relay Box |  |
| #2 | 0 | 0 | 0 | High | Read "1" from the FPGA EXT_SYN pin forced by USB Relay Box. | Value read by FPGA follows USB Relay Box voltage. LEMO connector is checked. |
| #3 | 0 | 0 | 0 | High | Read "1" from the FPGA EXT_SYN pin forced by USB Relay Box. |  |
| #4 | 0 | 0 | 1 | High | Read "0" from the FPGA EXT_SYN pin forced by termination pulldown. | Value read by FPGA is pulled low by termination no matter USB Relay Box voltage. Termination line is checked. |
| #5 | 1 | 0 | 0 | Low | Read "0" from the FPGA EXT_SYN pin forced by internal FPGA pulldown. IC1 output should be is Hi-Z. |  |
| #6 | 1 | 0 | 0 | High | Read "0" from the FPGA EXT_SYN pin forced by internal FPGA pulldown. IC1 output should be is Hi-Z. | No matter voltage forced by USB Relay Box value read by FPGA is constant. OEn line is checked. |
| #7 | 0 | 1 (output) | 0 | Low | Read "0" from the FPGA EXT_SYN pin forced by internal FPGA pulldown. IC1 output should be is Hi-Z. |  |
| #8 | 0 | 1 | 0 | High | Read "0" from the FPGA EXT_SYN pin forced by internal FPGA pulldown. IC1 output should be is Hi-Z. | IC1 buffer direction is changed to output, causing port connected with FPGA to go Hi-Z. FPGA IO is kept in Hi-Z state (with internal pulldown). If switching buffer direction didn’t work, value read from FPGA IO would follow USB Relay Box voltage (as shown in previous tests). EXT_SYNC_DIR line checked. |

Possible errors:

  - Bad connections to the USB relay box

  - Bidirectional buffer (IC1) might be badly soldered

  - LEMO port might be badly soldered

  - Faulty soldering of FMC connector

# Go to log directory (or extract zip file) ie:

cd ~/Desktop/pts_log/fmcmasterfip/adcdata

# Run python console:

python

# Import supplied functions

from read_samples import *

# Load data

[diff, pos, neg, time]=read_adc_samples('./data.bin')

# Plot data

show(time, diff)

# Plot two data sets

show2( time, pos, neg )

### Test 04

DS18B20 (IC11) thermometer verification.

- Read unique ID

- Verify ID correctness

- Take a temperature measurement

Possible errors:

- Get ID from DS18B20, DS18B20 ID verification, temperature measurement

  - Thermometer might be broken or badly soldered

  - Faulty soldering of FMC connector

### Test 05

24AA64T (IC10) serial EEPROM test.

- Write and verify some data patterns

- Generate and write following sdbfs files to EEPROM:

  - IPMI-FRU

    - vendor: CERN

    - name: FmcMasterFip

    - serial: <SERIAL>

    - part: EDA-03098-V1-<SPEED>

  - name

    - FmcMasterFip

  - speed

    - number from 0 (31.25k speed version) to  3 (5M speed version)

Possible errors:

- EEPROM communication, EEPROM content

  - EEPROM might be broken or badly soldered

  - Faulty soldering of FMC connector